FLSM WORKSHEET

Problem 1

- Major Network Address: 192.168.10.0/24

- Required Subnets: 4

- Custom Subnet Mask (CIDR): /26

- Custom Subnet Mask (Decimal): 255.255.255.192

- Usable Hosts per Subnet: 62

|  | Subnet-ids | First useable ip | Last useable ip | Broadcast Address |
| --- | --- | --- | --- | --- |
| 1 | 192.168.100.0 | 192.168.10.1 | 192.168.10.62 | 192.168.100.63 |
| 2 | 192.168.100.64 | 192.168.10.65 | 192.168.10.126 | 192.168.100.127 |
| 3 | 192.168.100.128 | 192.168.10.129 | 192.168.10.190 | 192.168.100.191 |
| 4 | 192.168.100.192 | 192.168.10.193 | 192.168.10.254 | 192.168.100.255 |

Problem 2

- Major Network Address: 200.15.4.0/24

- Required Subnets: 8

- Custom Subnet Mask (CIDR): /27

- Custom Subnet Mask (Decimal): 255.255.255.224

- Usable Hosts per Subnet: 30

|  | Subnet-ids | First useable ip | Last useable ip | Broadcast Address |
| --- | --- | --- | --- | --- |
| 1 | 200.15.4.0 | 200.15.4.1 | 200.15.4.30 | 200.15.4.31 |
| 2 | 200.15.4.32 | 200.15.4.33 | 200.15.4.62 | 200.15.4.63 |
| 3 | 200.15.4.64 | 200.15.4.65 | 200.15.4.94 | 200.15.4.95 |
| 4 | 200.15.4.96 | 200.15.4.97 | 200.15.4.126 | 200.15.4.127 |
| 5 | 200.15.4.128 | 200.15.4.129 | 200.15.4.158 | 200.15.4.159 |
| 6 | 200.15.4.160 | 200.15.4.161 | 200.15.4.190 | 200.15.4.191 |
| 7 | 200.15.4.192 | 200.15.4.193 | 200.15.4.222 | 200.15.4.223 |
| 8 | 200.15.4.224 | 200.15.4.225 | 200.15.4.254 | 200.15.4.255 |

Problem 3

- Major Network Address: 210.88.90.0 /24

- Subnet Mask: 28

- Custom CDR  255.255.255.240

- Block Size: 16

- Usable Hosts per Subnet: 14

|  | Subnet address | First useable ip | Last useable ip | Broadcast address |
| --- | --- | --- | --- | --- |
| 1 | 210.88.90.0 | 210.88.90.1 | 210.88.90.14 | 210.88.90.15 |
| 2 | 210.88.90.16 | 210.88.90.17 | 210.88.90.30 | 210.88.90.31 |
| 3 | 210.88.90.32 | 210.88.90.33 | 210.88.90.46 | 210.88.90.48 |
| 4 | 210.88.90.48 | 210.88.90.49 | 210.88.90.62 | 210.88.90.63 |
| 5 | 210.88.90.64 | 210.88.90.65 | 210.88.90.78 | 210.88.90.79 |
| 6 | 210.88.90.80 | 210.88.90.81 | 210.88.90.94 | 210.88.90.95 |
| 7 | 210.88.90.96 | 210.88.90.97 | 210.88.90.110 | 210.88.90.111 |
| 8 | 210.88.90.112 | 210.88.90.113 | 210.88.90.126 | 210.88.90.127 |
| 9 | 210.88.90.128 | 210.88.90.129 | 210.88.90.142 | 210.88.90.143 |
| 10 | 210.88.90.144 | 210.88.90.145 | 210.88.90.158 | 210.88.90.159 |
| 11 | 210.88.90.160 | 210.88.90.161 | 210.88.90.174 | 210.88.90.175 |
| 12 | 210.88.90.176 | 210.88.90.177 | 210.88.90.190 | 210.88.90.191 |
| 13 | 210.88.90.192 | 210.88.90.193 | 210.88.90.206 | 210.88.90.207 |
| 14 | 210.88.90.208 | 210.88.90.209 | 210.88.90.222 | 210.88.90.223 |
| 15 | 210.88.90.224 | 210.88.90.225 | 210.88.90.238 | 210.88.90.239 |
| 16 | 210.88.90.240 | 210.88.90.241 | 210.88.90.254 | 210.88.90.255 |

Problem 4

- Major Network Address: 195.40.50.0 /24

- Subnet Mask: /25 (255.255.255.128

- Block Size: 128

- Usable Hosts per Subnet: 126

|  | Subnet address | First useable ip | Last useable ip | Broadcast address |
| --- | --- | --- | --- | --- |
| 1 | 195.40.50.0 | 195.40.50.1 | 195.40.50.126 | 195.40.50.127 |
| 2 | 195.40.50.128 | 195.40.50.129 | 195.40.50.254 | 195.40.50.255 |

Problem 5

- Major Network Address: 172.16.0.0 /16

- Subnet Mask: /19 255.255.224.0

- Block Size: 32 (Third Octet)

- Usable Hosts per Subnet: 8,190

| Subnet # | Subnet Address | First Usable Host | Last Usable Host | Broadcast Address |
| --- | --- | --- | --- | --- |
| Subnet 0 | 172.16.0.0 | 172.16.0.1 | 172.16.31.254 | 172.16.31.255 |
| Subnet 1 | 172.16.32.0 | 172.16.32.1 | 172.16.63.254 | 172.16.63.255 |
| Subnet 2 | 172.16.64.0 | 172.16.64.1 | 172.16.95.254 | 172.16.95.255 |
| Subnet 3 | 172.16.96.0 | 172.16.96.1 | 172.16.127.254 | 172.16.127.255 |
| Subnet 4 | 172.16.128.0 | 172.16.128.1 | 172.16.159.254 | 172.16.159.255 |
| Subnet 5 | 172.16.160.0 | 172.16.160.1 | 172.16.191.254 | 172.16.191.255 |
| Subnet 6 | 172.16.192.0 | 172.16.192.1 | 172.16.223.254 | 172.16.223.255 |
| Subnet 7 | 172.16.224.0 | 172.16.224.1 | 172.16.255.254 | 172.16.255.255 |

Problem 6 (All 64 Subnets)

- Major Network Address: 150.50.0.0 /16

- Subnet Mask: /22 255.255.252.0

- Block Size: 4 (Third Octet)

- Usable Hosts per Subnet: 1,022

| Subnet # | Subnet Address | First Usable Host | Last Usable Host | Broadcast Address |
| --- | --- | --- | --- | --- |
| Subnet 0 | 150.50.0.0 | 150.50.0.1 | 150.50.3.254 | 150.50.3.255 |
| Subnet 1 | 150.50.4.0 | 150.50.4.1 | 150.50.7.254 | 150.50.7.255 |
| Subnet 2 | 150.50.8.0 | 150.50.8.1 | 150.50.11.254 | 150.50.11.255 |
| Subnet 3 | 150.50.12.0 | 150.50.12.1 | 150.50.15.254 | 150.50.15.255 |
| Subnet 4 | 150.50.16.0 | 150.50.16.1 | 150.50.19.254 | 150.50.19.255 |
| Subnets 5-59 | ...Increments by 4 | ... | ... | ... |
| Subnet 60 | 150.50.240.0 | 150.50.240.1 | 150.50.243.254 | 150.50.243.255 |
| Subnet 61 | 150.50.244.0 | 150.50.244.1 | 150.50.247.254 | 150.50.247.255 |
| Subnet 62 | 150.50.248.0 | 150.50.248.1 | 150.50.251.254 | 150.50.251.255 |
| Subnet 63 | 150.50.252.0 | 150.50.252.1 | 150.50.255.254 | 150.50.255.255 |

Problem 7 (All 512 Subnets)

- Major Network Address: 165.20.0.0 /16

- Subnet Mask: /25 255.255.255.128

- Block Size: 128 (Fourth Octet, alternates third octet)

- Usable Hosts per Subnet: 126

|  | Subnet Address | First Usable Host | Last Usable Host | Broadcast Address |
| --- | --- | --- | --- | --- |
| Subnet 0 | 165.20.0.0 | 165.20.0.1 | 165.20.0.126 | 165.20.0.127 |
| Subnet 1 | 165.20.0.128 | 165.20.0.129 | 165.20.0.254 | 165.20.0.255 |
| Subnet 2 | 165.20.1.0 | 165.20.1.1 | 165.20.1.126 | 165.20.1.127 |
| Subnet 3 | 165.20.1.128 | 165.20.1.129 | 165.20.1.254 | 165.20.1.255 |
| Subnets 4-509 | ...Increments fourth octet | ... | ... | ... |
| Subnet 510 | 165.20.255.0 | 165.20.255.1 | 165.20.255.126 | 165.20.255.127 |
| Subnet 511 | 165.20.255.128 | 165.20.255.129 | 165.20.255.254 | 165.20.255.255 |

Problem 8 (All 4 Subnets)

- Major Network Address: 10.0.0.0 /8

- Subnet Mask: /10 255.192.0.0

- Block Size: 64 (Second Octet)

- Usable Hosts per Subnet: 4,194,302

| Subnet # | Subnet Address | First Usable Host | Last Usable Host | Broadcast Address |
| --- | --- | --- | --- | --- |
| Subnet 0 | 10.0.0.0 | 10.0.0.1 | 10.63.255.254 | 10.63.255.255 |
| Subnet 1 | 10.64.0.0 | 10.64.0.1 | 10.127.255.254 | 10.127.255.255 |
| Subnet 2 | 10.128.0.0 | 10.128.0.1 | 10.191.255.254 | 10.191.255.255 |
| Subnet 3 | 10.192.0.0 | 10.192.0.1 | 10.255.255.254 | 10.255.255.255 |

Problem 9 (All 32 Subnets)

- Major Network Address: 24.0.0.0 /8

- Subnet Mask: /13 255.248.0.0

- Block Size: 8 (Second Octet)

- Usable Hosts per Subnet: 524,286

| Subnet # | Subnet Address | First Usable Host | Last Usable Host | Broadcast Address |
| --- | --- | --- | --- | --- |
| Subnet 0 | 24.0.0.0 | 24.0.0.1 | 24.7.255.254 | 24.7.255.255 |
| Subnet 1 | 24.8.0.0 | 24.8.0.1 | 24.15.255.254 | 24.15.255.255 |
| Subnet 2 | 24.16.0.0 | 24.16.0.1 | 24.23.255.254 | 24.23.255.255 |
| Subnet 3 | 24.24.0.0 | 24.24.0.1 | 24.31.255.254 | 24.31.255.255 |
| Subnets 4-29 | ...Increments by 8 | ... | ... | ... |
| Subnet 30 | 24.240.0.0 | 24.240.0.1 | 24.247.255.254 | 24.247.255.255 |
| Subnet 31 | 24.248.0.0 | 24.248.0.1 | 24.255.255.254 | 24.255.255.255 |

Problem 10 (All 1,024 Subnets)

- Major Network Address: 5.0.0.0 /8

- Subnet Mask: /18 255.255.192.0

- Block Size: 64 (Third Octet, loops through second octet)

- Usable Hosts per Subnet: 16,382

| Subnet # | Subnet Address | First Usable Host | Last Usable Host | Broadcast Address |
| --- | --- | --- | --- | --- |
| Subnet 0 | 5.0.0.0 | 5.0.0.1 | 5.0.63.254 | 5.0.63.255 |
| Subnet 1 | 5.0.64.0 | 5.0.64.1 | 5.0.127.254 | 5.0.127.255 |
| Subnet 2 | 5.0.128.0 | 5.0.128.1 | 5.0.191.254 | 5.0.191.255 |
| Subnet 3 | 5.0.192.0 | 5.0.192.1 | 5.0.255.254 | 5.0.255.255 |
| Subnet 4 | 5.1.0.0 | 5.1.0.1 | 5.1.63.254 | 5.1.63.255 |
| Subnets 5-1021 | ...Increments octets | ... | ... | ... |
| Subnet 1022 | 5.255.128.0 | 5.255.128.1 | 5.255.191.254 | 5.255.191.255 |
| Subnet 1023 | 5.255.192.0 | 5.255.192.1 | 5.255.255.254 | 5.255.255.255 |

These were the few practical examples related to flsm. Now we will see some practical enterprise level scenarios

VLSM DATASHEET

Scenario 1

Retail Branch Office (Store Location)

- Base Network: 192.168.10.0/24

- Total Usable Hosts Required: 154 hosts

| Subnet Name | Needed Hosts | Allocated Hosts | CIDR | Subnet Mask | Network Address | Usable IP Range | Broadcast Address |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Point-of-Sale (POS) Systems | 80 | 126 | /25 | 255.255.255.128 | 192.168.10.0 | 192.168.10.1 - 192.168.10.126 | 192.168.10.127 |
| Guest Wi-Fi | 45 | 62 | /26 | 255.255.255.192 | 192.168.10.128 | 192.168.10.129 - 192.168.10.190 | 192.168.10.191 |
| Store Management PCs | 20 | 30 | /27 | 255.255.255.224 | 192.168.10.192 | 192.168.10.193 - 192.168.10.222 | 192.168.10.223 |
| IP Security Cameras | 7 | 14 | /28 | 255.255.255.240 | 192.168.10.224 | 192.168.10.225 - 192.168.10.238 | 192.168.10.239 |
| Primary WAN to HQ | 2 | 2 | /30 | 255.255.255.252 | 192.168.10.240 | 192.168.10.241 - 192.168.10.242 | 192.168.10.243 |

2) Remote Engineering & R&D Hub

- Base Network: 192.168.55.0/24

- Total Usable Hosts Required: 101 hosts

| Subnet Name | Needed Hosts | Allocated Hosts | CIDR | Subnet Mask | Network Address | Usable IP Range | Broadcast Address |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Testing Workstations | 55 | 62 | /26 | 255.255.255.192 | 192.168.55.0 | 192.168.55.1 - 192.168.55.62 | 192.168.55.63 |
| Prototype IoT Lab | 28 | 30 | /27 | 255.255.255.224 | 192.168.55.64 | 192.168.55.65 - 192.168.55.94 | 192.168.55.95 |
| Hub Staff VoIP Phones | 12 | 14 | /28 | 255.255.255.240 | 192.168.55.96 | 192.168.55.97 - 192.168.55.110 | 192.168.55.111 |
| Local Print & NAS Storage | 4 | 6 | /29 | 255.255.255.248 | 192.168.55.128 | 192.168.55.129 - 192.168.55.134 | 192.168.55.135 |
| VPN Gateway Link | 2 | 2 | /30 | 255.255.255.252 | 192.168.55.136 | 192.168.55.137 - 192.168. | 192.168.55.139 |

Scenario 3: Bank Branch (High Security)

- Base Network: 192.168.200.0/24

- Total Usable Hosts Required: 76 hosts

| Subnet Name | Needed Hosts | Allocated Hosts | CIDR | Subnet Mask | Network Address | Usable IP Range | Broadcast Address |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Tellers & Customer Service | 40 | 62 | /26 | 255.255.255.192 | 192.168.200.0 | 192.168.200.1 - 192.168.200.62 | 192.168.200.63 |
| ATM Network (Segmented) | 14 | 14 | /28 | 255.255.255.240 | 192.168.200.64 | 192.168.200.65 - 192.168.200.78 | 192.168.200.79 |
| Vault & Alarm Systems | 6 | 6 | /29 | 255.255.255.248 | 192.168.200.80 | 192.168.200.81 - 192.168.200.86 | 192.168.200.87 |
| Branch Manager Office | 4 | 6 | /29 | 255.255.255.248 | 192.168.200.88 | 192.168.200.89 - 192.168.200.94 | 192.168.200.95 |
| SD-WAN Router Interlink | 2 | 2 | /30 | 255.255.255.252 | 192.168.200.96 | 192.168.200.97 - 192.168.200.98 | 192.168.200.99 |

Scenario 4: Localized Data Center DMZ Zone

- Base Network: 203.0.113.0/24 (Public Class C range)

- Total Usable Hosts Required: 53 hosts

| Subnet Name | Needed Hosts | Allocated Hosts | CIDR | Subnet Mask | Network Address | Usable IP Range | Broadcast Address |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Web Server Farm | 30 | 30 | /27 | 255.255.255.224 | 203.0.113.0 | 203.0.113.1 - 203.0.113.30 | 203.0.113.31 |
| External Reverse Proxies | 10 | 14 | /28 | 255.255.255.240 | 203.0.113.32 | 203.0.113.33 - 203.0.113.46 | 203.0.113.47 |
| Public DNS Servers | 5 | 6 | /29 | 255.255.255.248 | 203.0.113.48 | 203.0.113.49 - 203.0.113.54 | 203.0.113.55 |
| Firewall HA Heartbeat | 2 | 2 | /30 | 255.255.255.252 | 203.0.113.56 | 203.0.113.57 - 203.0.113.58 | 203.0.113.59 |
| Load Balancer Virtual IPs | 2 | 2 | /30 | 255.255.255.252 | 203.0.113.60 | 203.0.113.61 - 203.0.113.62 | 203.0.113.63 |

Scenario 5:

Logistics Warehouse & Distribution Site

- Base Network: 192.168.150.0/24

- Total Usable Hosts Required: 192 hosts

| Subnet Name | Needed Hosts | Allocated Hosts | CIDR | Subnet Mask | Network Address | Usable IP Range | Broadcast Address |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Handheld Wi-Fi Scanners | 115 | 126 | /25 | 255.255.255.128 | 192.168.150.0 | 192.168.150.1 - 192.168.150.126 | 192.168.150.127 |
| Shipping Admin Offices | 50 | 62 | /26 | 255.255.255.192 | 192.168.150.128 | 192.168.150.129 - 192.168.150.190 | 192.168.150.191 |
| Inventory Label Printers | 12 | 14 | /28 | 255.255.255.240 | 192.168.150.192 | 192.168.150.193 - 192.168.150.206 | 192.168.150.207 |
| Climate Control/HVAC IoT | 5 | 6 | /29 | 255.255.255.248 | 192.168.150.208 | 192.168.150.209 - 192.168.150.214 | 192.168.150.215 |
| Main Site Gateway Link | 2 | 2 | /30 | 255.255.255.252 | 192.168.150.216 | 192.168.150.217 - 192.16 |  |